 [image: image6.png]Hytten

BIENE



                                            An ISO9001  Company

	

	LXSG-15-20MM Multi-jet Vane Wheel Dry-dial Cold(hot) water meter.

                                [image: image1.png]



Application:

--Measuring the total volume of cold(hot) water passing through the pipeline.
-NOTE:This type of water meter can be used for a reading transmission system as equipped with a bulit-in-sensor.
Feature:
--Multi-jet, dry-dial; Magnetic drive.
--Vacuum sealed register ensures the dial keep free from fog and frost, Keep the reading clear in a long term service;
--Removable measuring unit,easy installation and maintenance, Anti-magnetic function.
--Brass body,Out-adjust,Small volume and light weight.
-- Technical data conform to international standard ISO4064 Class B
Working condition:

Water temperature: T30 T50 T90
 Water pressure class: MAP16  MAP10
 Pressure-loss Class: △p63
Maximum permissible error:

(1) From minimum flow-rate(Q1) inclusive to transitional flow-rate(Q2) exclusive±5%.
(2) From transitional flow-rate(Q2) to overload flow-rate(Q4):±2%;
(Hot water meter ±3%)
MAIN TECHNICAL SPECIFICATIONS(R80/100)
Type
Size   
Class
Q4
Overload                Flow
Q3
Permanent          Flow
Q2
Transitional Flow
Q1
Minimum
Flow
Minimum
Reading
Maximum
Reading
m3/h
m3
LXSG-15
½"
R80
(R100)
3.125
2.5 
0.05 
0.03125
0.00005
99,999
LXSG-20
¾"
5
4 
0.08 
0.05
0.00005
99,999
   DIMENSIONS 
Type
Size
L           Length
B        Width
H             Height
Connecting Thread
mm
d
LXSG-15m
½"
165
89
98
G3/4
LXSG-20m
¾"
190
89
98
G1
[image: image2.jpg]



[image: image3.png]Error(%)

Flow Error Curve

QT

mEREML

Flow Rate (m7h)



        [image: image4.png]Pressure Loss (Mpa)

Pressure Loss Curve

0063
0,04
A
0025
0.01
=
0 25 3

E AR £

Flow Rate (m’h)



         


[image: image5.jpg]24 23 22 21

______




	LXSG-15M-20M  MULTI-JET, VANE WHEEL, DRY-DIAL WATER METER

	Code
	Description
	Material
	QTY

	1
	Lid
	HDPE
	1

	2
	Hinge Pin
	Stainless
	1

	3
	Cap
	Brass
	1

	4
	Sliding Gasket
	PP
	1

	5
	Glass Fixing Ring
	ABS
	1

	6
	Glass
	Tempered Glass
	1

	7
	O-ring
	 Silicon Rubber
	1

	8
	Register Fixing Ring
	ABS
	1

	9
	Register
	Assembly
	1

	10
	Central Gear
	Component
	1

	11
	Magnetic Shield Ring
	Iron
	1

	12
	Gearing box
	ABS Plastic
	1

	13
	Tightening Gasket
	Rubber
	1

	14
	Turbine Component
	Component
	1

	15
	Measuring Chamber
	HIPS
	1

	16
	Bottom Strainer
	LDPE
	1

	17
	Body
	Brass
	1

	18
	Adjusting Screw
	POM
	1

	19
	Adjusting Gasket
	Nylon
	1

	20
	Adjusting Plug
	Brass
	1

	21
	Inlet Strainer
	PP
	1

	22
	Coupling Gasket
	Rubber
	2

	23
	Coupling Tail Piece
	Brass
	2

	24
	Coupling Nut
	Brass
	2


LIANYUNGANG HYTTEN METER CO.,LTD.  

 Add:No. 8 Hanguo Industrial Development Zone,Luoyang Village 

Dunshang Town,Ganyu District, Lianyungang, Jiangsu, China

Tel:      ++86-518-85907891    
Fax:     ++86-518-85907890             Postal code:   222000
E-mail: hytten@hyttenmeter.com
http://www.hyttenmeter.com    www.hyttenwatermeter.com 
----------------------------------------------------------------------------------------------------------------------
LIANYUNGANG HYTTEN METER CO.,LTD.           0086-518-85907891

[image: image6.png]